Audio

Item; Wire Color; Polarity; Wire Location. Radio 12V; red/white; +; radio, white 20 pin plug, pin 10. Radio Ground; black; -; radio, white 20 pin plug, pin 20. Radio Ignition; yellow; +; radio, white 20 pin plug, pin 1. Radio Illumination; green/white (dimmer); +; radio, white 20 pin plug, pin 9. Factory Amp Turn-on; pink or blue; +; radio, white 18 pin plug, pin 17 or 7. Power Antenna; pink; +; radio, white 20 pin plug, pin 14. Left Front Speaker (+/-); white/blue - black/yellow; +,-; radio, white 20 pin plug, pins 8 and 18. If equipped, the amplifier is under the driver seat. On vehicles with amplifier, the speaker wires are white/blue - black/yellow at the amplifier, white 18 pin plug, pins 8 and 18. The tweeter wires are white/blue - black/yellow at the amplifier, white 14 pin plug, pins 6 and 14... Right Front Speaker (+/-); white/red - black/red; +,-; radio, white 20 pin plug, pins 6 and 16. If equipped, the amplifier is under the driver seat. On vehicles with amplifier, the speaker wires are white/red - black/red at the amplifier, white 18 pin plug, pins 7 and 17. The tweeter wires are white/red - black/red at the amplifier, white 14 pin plug, pins 5 and 13... Left Rear Speaker (+/-); yellow/blue - gray/blue; +,-; radio, white 20 pin plug, pins 7 and 17. If equipped, the amplifier is under the driver seat. On vehicles with amplifier, the speaker wires are yellow/blue - gray/blue at the amplifier, white 14 pin plug, pins 1 and 7... Right Rear Speaker (+/-); yellow/red - gray/red; +,-; radio, white 20 pin plug, pins 5 and 15. If equipped, the amplifier is under the driver seat. On vehicles with amplifier, the speaker wires are yellow/red - gray/red at the amplifier, white 14 pin plug, pins 2 and 8... Center Channel (+/-); N/A. Subwoofer (+/-); green - yellow (1), blue - white (2); +,-; amplifier, white 18 pin plug, pins 1 and 9 (1), 2 and 10 (2). If equipped, the amplifier is under the driver seat... Aux. Audio Input Left (+/-); blue/white - black/wht (common) (Lancer Evolution); +,-; radio, white 18 pin plug, pins 13 and 14. Aux. Audio Input Right (+/-); purple - black/white (common) (Lancer Evolution); +,-; radio, white 18 pin plug, pins 4 and 14. RSE Video (+/-); N/A. RSE Audio Left (+/-); N/A. RSE Audio Right (+/-); N/A. Satellite Radio 12 Volts; red/white; +; Satellite Radio Tuner, white 14 pin plug, pin 1. If equipped, the Satellite Radio Tuner is behind the glove box... Satellite Radio Ground; black; -; Satellite Radio Tuner, white 14 pin plug, pin 8. Satellite Radio Ignition; (CAN bus). Satellite Radio Antenna; (coaxial cable). Satellite Audio Left (+/-); yellow - blue (common); +,-; Satellite Radio Tuner, wht 14 pin plug, pins 6, 7. Satellite Audio Right (+/-); brown - blue (common); +,-; Satellite Radio Tuner, wht 14 pin plg, pins 14, 7.